ELBRUS

A telescope pointing program

Alfonso Pulido

(Written 15-6-2005)

(Updated 04-February-2008)

***********************************************************************************

What it does:

Elbrus analyzes a sky image and calculates the coordinates of the image centre.

What for ?

In astrophotography, a common problem is putting the object inside the ccd frame. Often we are near the target, we have an image with stars but we don't know in which direction is the object. The principal aim of this program is to help in such a situation. If we know exactly the coordinates of the image centre, it will be easy to drive the mount to the object.

Another situation where Elbrus is useful is when we are far from the telescope ( remote control ) and we don't know where is the tube pointing to. In such a 'lost in space ' case, we can find our position by doing a search in a wide region such as a spherical ring or even the full sky.

Elbrus is also successfully used for recentering the scope between captures. Each image is analysed and the offset from the reference one is computed. This offset is used by the mount driving program to recenter the mount. This way we can capture images (for example images of one minute exposure each) during hours in despite of the drift in the telescope tracking.

How does it work ?

The first thing to do is to extract the stars from the image. A scan gives us the contours with intensities above the "brigthness threshold". Then the contours with perimeter less than a limit are selected. This way too large objects are excluded. Finally we calculate the intensity of the candidate stars and we conserve the strongest ones. Further on we shall deal only with these stars.

Once the stars are selected, the distances between them are calculated, and these distances are fetched in the distances database. The set of found distances is used to build all the possible polygons until the maximal-sides-polygon is found. If a polygon with more than  15 sides is found, the solution is offered to the user.

INTRODUCTION:

IMPORTANT NOTES:

+++  In order to properly use this program ,it is imperative to calibrate it for your images (see why in CALIBRATION below ).

If Elbrus is not calibrated for your own pixel size and image orientation, you will get absolutely no valid results.

The reason is that we calculate the angular distances between stars. And this needs a good precision knowledge of pixel size.

+++  The distances database is optimised for an image size of about 10 x 15 arc minutes. Nevertheless it works perfectly with sizes of  20 x 30 minutes. I also made successful tries with 1 x 1.5 degrees images.

+++  Only FITS, BMP and JPG image files are accepted.

WORKING  MODES:

There are basically two working modes.

The first is the so called "Manual mode". You have to click on the 'Analyze' button to begin the analysis (also from the drop down menu or by pressing the space bar).

The second mode is the "Waiting mode".

Here the images are analyzed automatically as they are recorded in the image folder.

When we are in the 'D-Wait for image' mode, the image is analyzed when arriving in the 'Auto image' folder. When in the 'E-Wait for message' mode, the image indicated in the message file elbrus.txt should be analyzed. The text file elbrus.txt must be created by the capture program or another external way.

Using Elbrus in the Manual Mode:

In this working mode, you have to give to the program the name of the image you want to analyze.

Go to the  'Mode' option and choose one of the following posibilities:

A-Image path.

B-Most recent image.

C-Image from last message.

Then in the  'Image' option choose which part of the image must be taken into account. If needed, mark the 'Reversed image' option.

Go to the 'Coordinates' option and select the source for the searching center coordinates,

More precisely:

-coordinates from the FITS file header,

-manually introduced coordinates,

-from an ASCOM telescope,

-coordinates of the  Sky Chart program by DDE,

-coordinates recieved by DDE from an external program.

In the 'Search area' option, select a sky window for the search in the database. Por example:

- 5 x 5 degrees field,

Push the 'Analyze' button. That's all.

Using Elbrus in the 'D - Wait for an image' Mode:

Go to the 'Coordinates' option and select the searching center coordinates.

Go to the  'Mode' option and choose:

D - Wait for an image

From now, the program will analyze every new image added to the folder 'Auto images'. ( see 'Edit parameters' option in 'Others'). The new image file must have been created after you choosed this option.

Using Elbrus in the  'E - Wait for a message':

Go to the 'Coordinates' option and select the searching center coordinates.

Go to the  'Mode' option and choose:

E - Wait for a message

From now, the program will read every new elbrus.txt file added to the folder 'Auto images'. ( see 'Edit parameters' option in 'Others').

This elbrus.txt text file with the name of the image, must be created by the capture program or by script.

Following every image analysis, Elbrus creates a text file named "elbrus.pos" with the coordinates of the image centre. This file can be used by another program to control the telescope mount position.

INTERNET:

You can download the last program version and get more help in the Yahoo group:

in english http://tech.groups.yahoo.com/group/StarLocatorElbrus,

in french http://fr.groups.yahoo.com/group/BoussoleStellaireElbrus,

in spanish http://es.groups.yahoo.com/group/LocalizaEstrellas.

For general information see also the web page:

http://astrosurf.com/pulgar/elbrus/elbrusin.htm

THE MAIN WINDOW CONTROLS:

In the main window you can find all the nesessary controls to operate this program during an observing session.

The tittle bar:

In the tittle bar, you can see the search status. At the end of the analysis,  the result with the centre coordinates in degrees will be displayed. The final polygon sides number and the distance in degrees from the target are also shown. The number of sides of the polygon-solution is the main criterion in measuring the goodness of the solution.

Options menu:

Contains the program options. See the details below.

The 'Analyze' button:

Launches the scanning and analysis of the image. You can use the space bar too.

The 'Centre' box:

Shows the image coordinates when found (here the RA is in hours, minutes and the declination in degrees, minutes). If the polygon has less than 15 sides, the text displayed will be red color. For 15 and more sides, the text will appear in blue color.

The image box:

It's where the main image is shown. We'll find also displayed the information about:

- the found contours (blue ink),

- selected stars (in a red circle),

- white lines representing the distances found in the database,

- red lines representing calculated distances,

- blue lines for calculated distances from candidate stars,

- GSC numbers for the identified stars,

- astronomical objects in the image,

- the target point and the compass.

The visualization scroll bars:

Used for a better visualisation. The button on the left is for automatic stretching.

The mouse pointer position boxes:

When moving the mouse on the image, the position of the mouse and the pixel intensity are shown. When a solution for the image has been found, the RA, DEC coordinates of the pixel are also shown.

The Threshold bar:

The THRESHOLD bar modifies the value of the intensity threshold used to determine whether a pixel belongs to a star. These pixels will be analyzed later more slowly. Allowed values run from 0,01 to 0,90. A 0 value is equivalent to the backgroud level. A value of 1 is equivalent to the maximal intensity in the image.

When the check box "Auto" is checked, the threshold  is selected in an automatic way. This is the normal working mode.

THE MAIN MENU OPTIONS:

Mode:

It serves to indicate where the image name is taken from.

Origin of the image for manual modes:

A-Image path. To choose a specific image manually.

B-Most recent image. When we check this option, every time we click on the 'Analyze' button, the program will analyze the most recent image from the image folder.

C-Image from last message. Every time we click on the 'Analyze' button, the program will read the most recent message already in the image folder. Then it analyzes the image whose path is in this message.

Origin of the image for automatic modes:

D-Wait for image. The program enters a waiting state. Every new image added to the image folder will be automatically analyzed.

E-Wait for message. The program enters a waiting state. Every new message added to the image folder will be automatically read. Then Elbrus will analyze the image whose path is in the file message elbrus.txt. Of course the camera capture program must create the elbrus.txt file. Elbrus creates a file  elbrus.pos with the image centre coordinates. This way the telescope control program can send the corrections to the mount.

Exit:

The parameters are saved and Elbrus is closed.

Image:

Which part of the image:

It determines which part of the image will be analyzed. There are three possibilities:

Analyze the full image.

Analyze inside the rectangle.

Analyze outside the rectangle.

See The analysis rectangle below.

Coordinates:

Here we indicate the coordinates around which we are looking for a solution (the searching center):

from the FITS header: check this option for a FITS type file image with aproximative coordinates in the header ( OBJCTRA and OBJCTDEC or  RA and DEC).

manually selected …: select the searching centre manually in the combos.

from an ASCOM telescope: the ASCOM plateform is used to obtain the coordinates of the point the scope is supposed to be pointing to.

from Sky Chart by DDE: the searching center will be the point where Sky Chart program is pointing to. DDE communication is used here.

received by DDE: the coordinates received by DDE from an external program (see bellow "Communication with other programs by DDE" ).

Search field:

We indicate here the area of  the sky where we are looking for a solution :

For manual and automatic modes:

5x5 degrees field with fixed image angle: this is the normal searching area,

5x5 degrees field with dual image angle: For GEM type mount you don't have to care about the two 
positions facing East and facing West of the tube .

2x2 degrees field, any image angle: the search is done in a little 2 by 2 degrees sky region, but the 'image angle' parameter will take all possible values. This may be useful for ALT-AZ mounts.

40 x 40 minutes field, any image angle: same use.

For manual modes only:

along the meridian: search in a 5 degrees wide tape. Scans along declination

along the parallel of latitude: search in a 5 degrees height tape. Scans along AR.

30 x 30 degrees field:  use this option when there is a big indetermination,

in a spherical ring: use this option when you know more or less the pointing declination of the telescope, and have no idea about the RA.

search the full sky: all the sky database should be searched. This may last several minutes in a slow computer.

Show:

Check or uncheck for viewing GSC numbers, lines, objects, target point.

Synchronize:

This option is enhabled when a valid solution has been found. The found image centre coordinates are sent to the program that drives the telescope mount.

Synchronize the telescope:

the scope will be synchronised with the image coordinates.

Telescope type:

you must first choose a telescope communication mode:

-   ASCOM telescope

for telescope communication using the ASCOM platform

-   Sky Chart by DDE

for Sky Chart (Cartes du Ciel)  by DDE. At the same time, the telescope control program PicGoto  will be syncronised by DDE.

-   DDE telescope

for others programs communicating with Elbrus by DDE. See "Communication with others programs"

Update the centre of the search (combos):

The searching centre coordinates should be updated to the image centre coordinates.

This option is very useful when searching in a spherical ring or in the full sky. In both cases, if we don't update in this way the searching centre values, they will be loosen and we 'll have to repeat the search.

Others:

Contains several not frequently used options.

Edit parameters:

For modifing the working parameters.

See details in THE PARAMETERS WINDOW below.

Load  parameter's profile:

The parameters from an existing file are used.

Save  parameter's profile:

The actual parameters are saved to a file.

Search summary:   Shows a summary of the scan and search prosessus. The 'Save elbrus.log file' button saves this summary into the 'Images' folder.

Calibrate the pixel size and image angle:  For program calibration. See more details below in THE CALIBRATION.

Fine adjusting the image angle: To readjust the image angle. This is done automatically using the last solution found.

Help in calibrating the image angle: Use it when you know with good exactitude the pixel size, but  you don't know the image angle exactly. Here an image solution is searched by steping the image angle from -15 degrees to +15 degrees around the supposed image angle.

Choose an ASCOM telescope:  Select the telescope driver when using the ASCOM plateform for communicating with the telescope.

Analyze just for recentering:  Elbrus can be used for mesuring the relative shift between two images in order to recenter the telescope.When this option is selected no time is wasted in searching the image coordinates. The program will just calculate the image shift and show it in the text file elbrus.pos. This shift can be then  used by a mount-driving program or script to recenter the scope between captures.

Reference image for recentering:  Click here to indicate that the present image is the reference image. Anyway, every image named "referen.fit" or "referen.bmp" is automatically taken as a reference image.

Language: Three options: english, french and spanish.

Help: Contains a very short help text.

THE DROP-DOWN MENU:

When right button clicking on the image,a drop down menu appears with the following options:

Analyze the full image. See The analysis rectangle below.

Analyze inside the rectangle.

Analyze outside the rectangle.

Star A for calibration. See THE CALIBRATION WINDOW below.

Star B for calibration.

Calibrate pixel size and image angle.

Fine adjusting the image angle.

The analysis rectangle:

This rectangle defines the image zone from where the stars should be peaked up for the analysis.

By moving the mouse over the image with the left button down, we can draw a rectangle. This way we define a region inside the rectangle and a region outside the rectangle.

We can choose the full image, the inner region of the rectangle or the outer region of the rectangle. This may be very useful in order to exclude a very populated zone. Also it may be useful when working with big field images: by taking roughly a 10 x 15 minutes rectangle , we shall be in the best conditions for the database.

THE CALIBRATION:

Calibrating means  giving values to the "Image angle" and the "Arcsec per pixel" parameters. These parameters are crucial in the process of successfully extracting from the enormous database only those distances that fall within the precision range.

You can type manually these values in the 'Others', 'Edit parameters' option.  Also you can let Elbrus calculate them from the 'Others', 'Calibrate pixel size and image angle' or use the 'Help in calibrating the image angle'.

Manual calibration:

Take note that these values have to be exactly determined. The "Arcsec per pixel" must have a precision better than  1 per cent. The "Image angle" must be given with a precision better than one degree.

Whenever you get a succesful image solution with the manual calibration, it is strongly recommended to do a second calibration from 'Others', 'Calibrate pixel size and image angle'. In this way the parameters will be calculated more exactly.

The calibration window:

When calibrating in the 'Others', 'Calibrate pixel size and image angle' option, we must begin from an image with recognizable stars. That means we must have two stars with known GSC number.

Then we click on 'Analyze'. The image is scanned, but obviously ( not yet calibrated ) no valid solution is found. We must check that at least two stars have been selected ( a red circle arround them ). Now we are going to calibrate using two of these selected stars and their GSC numbers. It is preferable to choose two distant stars in the image.

Put the mouse arrow on the first star and press the right button to get the drop down menu. Select the option 'Star A to calibrate'.

Put the mouse arrow on the second star and press the right button to get the drop down menu. Select the option 'Star B to calibrate'.

Go to the 'Others', 'Calibrate pixel size and image angle' option. In the calibration window type the GSC numbers of the two stars. Press "Calibrate". To accept the calculated parameters click on "Apply calibration". From now on, Elbrus is calibrated and ready for use.

Fine adjusting the image angle:

With this option the parameter "Image angle" is recalculated using the last solution found. This may be useful when doing an important declination slew with a deficiently Polar adjusted mount.

It is also useful after removing and replacing back the camera, to correct for a little orientation change.

Help in calibrating the image angle:

It is not always aisy to recognize stars in an image, and therefore an other method is proposed  here.

First you have to calculate the pixel size in arcsecods. That's aisy using the focal distance of your telescope, the size of the CCD pixel and the binning used  when capturing the image.

Then you must evaluate the angle of the image. You only need an approximate value, the program will scan +- 15 degrees.

Now go to :

Option  'Others',  'Edit parameters' and in  "Arcsec per pixel" type the calculated pixel size. In the "Image angle in degrees" type the approximate value.

In the option  'Coordinates', '-manually selected…' choose the aproximates coordinates of the image. As the program will search in a 30 x 30 sky window, so you don't need a big accuracy.

Finally go to 'Others' ,  ' Help in calibrating the image angle', verify all the entries, and launch the search of the image angle around the supposed approximate value.

This method is very powerful whenever you know with a good precision the size of image pixels, and this is aisy enough.

The two other parameters (image angle and image center) don't need a big precision, as far as the program seeks them within 30 degres .

THE PARAMETERS WINDOW:

Optics:

Image angle in degrees: This is the angle between the image vertical  and the North direction. If your image is oriented with the North up and the East at the left, then this angle is zero.

Arcsec per pixel: This is the angular size of an image pixel.

Reversed image:

Check this option when the image is reversed. The image is not reversed if  you walk around the  North, West, South, East, in a clockwise direction. For example when the  North is up and the West is at the right hand, then the image is not reversed.

Stop searching when solution with more than … sides:  This applies only when searching in a

spherical ring or in the full sky. This way the search is less time consuming.

No error messages.  No error messageBox will be produced by the program. Useful while in a "Wait for"  mode of operation during unattended runs.

Synchronize the telescope automatically:

When this option is checked, every time a solution-polygon with more than indicated sides number is found, the program will automatically synchronize the telescope choosen in the 'Synchronize'/'Telescope type' option.

Synchronize with current equinox coordinates:

The current equinox coordinates are sent to the telescope. Otherwise the 2000 coordinates are used when synchronizing.

WCS:

Adds the WCS coordinates to the FITS header, when  the image was solved with more sides than those indicated latter in this same line.

Folders:

Images:  for the images and messages in the "Wait for" modes.

Stars DB: the folder with the stars database est.bin, and the objects database pgc.bin.

Distances DB: the folder with the distances database( files anil000.bin …. ani179.bin ).

The 'Show the advanced parameters' button: gives access to some parameters those modification is not recommended.

Positioning:

Distances percent error:  only are selected from the database the distances within plus or minus this error. A good value is 2 per cent.

Inclination percent error: only are selected from the database the distances within plus or minus this error. A good value is 1 degree.

Searching window:

These are the width and the height of the sky window where the search in the distances database will be done. A good value is 5 by 5 degrees.

The PERIMETER bar modifies the maximal perimeter for the contours (from  10 to 100 pixels). Greater perimeters should be discarded.

The STARS bar allows you to choose the number of stars that will be selected for the search  (from 5 to 10 stars).

THE ANALYSIS RESULTS:

When the analysis of the image is ended and no polygon with more than 4 sides is found, the text "Position not found" is displayed in the 'Centre' box.

When a polygon with enough sides is found, the result is diplayed in the tittle bar: the image centre given in degrees, the number of sides of the polygon-solution, and the distance from the searching centre in degrees. The sides number is the master criterion to determine the goodness of the found solution.

In the box 'Centre' are displayed the coordinates of the image centre ( here the RA is expressed in hours ,minutes ). The text color is red when the polygon-solution has less than 15 sides. The text color is blue when the polygon-solution has 15 or more sides. All coordinates are given in the J2000 reference frame.

In the image box, over the main image are displayed:

- the found contours (blue ink),

- selected stars (in a red circle),

- white lines representing the distances found in the database,

- red lines representing calculated distances not found in the database,

- blue lines representing supplementary calculated distances,

- GSC numbers for the identified stars,

- astronomical objects in the image,

- the target point and the compass.

At the end of every image analysis Elbrus generates a text file "elbrus.pos" with the found image centre coordinates. This file may be used by another programs to control the telescope mount.

See also " COMMUNICATION WITH OTHER PROGRAMS BY DDE" below.

THE SEARCH SUMMARY WINDOW:

After every image analysis, the option 'Others' , 'Search summary' gives us a resumé of the scanning and seeking process. This information can be helpfull when no solution has been found.

THE FILES AND THE FOLDERS:

The program file:

You must put the next elements in the same folder:

the program elbrus.exe,

the text file param.txt that contains the working values for the parameters used by the program. the file elbrus.dll, a dll for accelerating some calculations.

The folder with the distances database

It contains 180 files Anil000.bin …. Ani179.bin,  pole000.bin and pole179.bin with precalculated distances. The size of this database is nearly 800 MB.

The folder for the stars database

Place here the file est.bin with the coordinates of the used stars, and

the file pgc.bin  with the coordinates of some galaxies, planetary nebualaes, etc. ( does not contain open clusters).

The mailbox folder for file interchanging

This is were Elbrus writes the text file "elbrus.pos" with information about the found solution.

And where it looks for a text file "elbrus.txt" with the name of the image to analyze. Used only in "Wait for" mode.

ASCOM COMMUNICATION WITH OTHERS PROGRAMS

Elbrus can communicate with other programs using the ASCOM plateform. This way , it can obtain the supposed coordinates of the telescope, and then synchronize the telescope with the image coordinates.

See the options

- 'Others' / 'Choose an ASCOM telescope…'

- 'Synchronize' / 'ASCOM telescope'

- 'Coordinates' / ' from an ASCOM telescope'

COMMUNICATION WITH OTHER PROGRAMS BY DDE

Elbrus can communicate with other programs by  Dynamic Data Exchange (DDE).

At present  Sky Chart ( Cartes du Ciel) and PicGoto are synchronized by DDE when clicking  'Synchronize the telescope'. The COR (remote control sytem ) synchronizes by exchanging text files.

You can find detailed information about these programs at their web pages:

COR  by Cristóbal García                           http://personal.auna.com/cristog/cor/cor.htm

Cartes du Ciel  by Patrick Chevalley          http://www.stargazing.net/astropc/index.html

PicGoto by Angel Caparrós                        http://www.angelcaparros.es/

Note for programmers:

To send or receive data from Elbrus, the client DDE program must use the following DDE parameters:

DDE server in Elbrus:

linkTopic = elbrus|formExtrae

linkItem = labelDDE

Clicking 'Synchronize the telescope' will update labelDDE with the format:

If coordinatesOK Then

text = "ALI AR=" & Format(raFin * 100000, "00000000") & " DE=" & Format(deFin * 100000,

"0000000") & " "

If text = Me.LabelDDE Then ' the same message

Me.LabelDDE = text & "+" >

Else

Me.LabelDDE = text

End if

End If

arFin and deFin in degrees.

Other programs can send to Elbrus the telescope position by DDE. Upon receiving them, Elbrus will paint a green circle on the image, indicating where the telescope is pointing to. The recognized format is:

"CEN RA=" & Format(arTarget * 100000, "00000000") & " DE=" & Format(deTarget * 100000, "0000000") & " "
arTarget and deTarget in degrees.

These received coordinates shall be used as searching center when the option 'Coordinates', 'received by DDE' is selected.

COMMUNICATION BY TEXT FILES

Elbrus can communicate with other programs by exchanging plain text files.

Sending to Elbrus the path of an image file to analyze:

You have to write a text file named elbrus.txt into the "Images" folder (see Parameters).

This elbrus.txt file must have 20 text lines with the following format:

(please don't modify the comments in odd lines )

1.- image file path:

c:\elbrus\imacris\m4i.fit      this is an example

2.- Coordinates in the FITS header (1 = yes, 0 = no)

1   this is an example

3.- RA searching centre:

246  this is an example

4.- DEC searching centre:

-26,32  this is an example

5.- not used 5

5

6.- not used 6

6

7.- not used 7

7

8.- not used 8

8

9.- not used 9

9

10.- not used 10

10

Sending to Elbrus the path of a parameters file to be used further on:

You have to write a text file named elbrus.txt into the "Images" folder (see Parameters).

This  elbrus.txt file must have 20 text lines with the following format:

(please don't modify the comments in odd lines )

1.- parameters file path:

c:\elbrus\imacris\paramCris.txt      this is an example

2.- not used 6

2

3.- not used 6

3

4.- not used 6

4

5.- not used 5

5

6.- not used 6

6

7.- not used 7

7

8.- not used 8

8

9.- not used 9

9

10.- not used 10

10

Receiving from Elbrus the coordinates of the image centre:

You have to wait for a text file named elbrus.pos in the "Images" folder (see Parameters).

This  elbrus.pos file has several text lines. The two first lines contain the image center coordinates.

024 sides RA= 051.456 DE= +82.468 RA= 03h25.8m DE= +82º28' 28/11/07 17:55:53 J2000  coor

024 sides RA= 051.819 DE= +82.495 RA= 03h27.3m DE= +82º29' 28/11/07 17:55:53 actual coor

The first number is the number of the polygon-solution sides. This number is the criterion you must use for accepting the goodness of the result.

LIMITATIONS:

- There are 'holes' without stars in the database. For example for M4, M33, …When building the databases, I used the Thycho and the GSC star catalogs. Normally these holes correspond to well visible,  bright objects, so this is not a big problem when pointing the telescope.

- The full distances database for the full sky occupies more or less 800 megabytes. Nevertheless the database is organized in 180 files, one for each declination degree. So it is possible to work with a part of the database. Of course the images must correspond to that part of the database.

Elbrus gives a solution when a more than 5 sides polygon is found.

Preferably trust in solutions with more than 14 sides.

Upon now no wrong solution was found with more than 10 sides polygon.

It is the user responsability to determine when a solution is good enough.

IF ELBRUS CANNOT FIND A SOLUTION:

If the number of selected stars is too low:

decrease the threshold value

If too many contours:

increase the threshold value

If there is a region in the image with too close selected stars, try excluding this region with a rectangle (see The analysis rectangle ) and then  'Image', 'Analyze outside the rectangle'.

When in spite of these points , no solution is found, up to now the problem was:

-  wrong searching coordinates (most frequent reason),

-  reversed image,

-  the image was captured with a different optics ( and as a consequence, the pixel size is different).

-  few selected stars (it is preferable to use 10 stars).

-  no or wrong calibration. See THE CALIBRATION.

REMEMBER:

Read several times this help text.

You can get more help in the Yahoo group:

in english http://tech.groups.yahoo.com/group/StarLocatorElbrus,

in french http://fr.groups.yahoo.com/group/BoussoleStellaireElbrus,

in spanish http://es.groups.yahoo.com/group/LocalizaEstrellas.

Technical specifications of Elbrus:

Program type:  Star locator.

Object: Help in telescope pointing from a captured image.

Strategy: Search in the proprietary database, by distances between stars and image orientation.

Building the polygon with more sides.

At the program entry: An image with stars.

Size of the image:  from 10x10  to  20x30 arc minutes.

Image types: FITS, BMP and JPG.

At the program output: Image center coordinates,

GSC numbers of the identified stars, main identified astronomical objects, distance to the target, direction to the target.

Languages: English, french, spanish.

Working modes: manual and automatic.

Image analyse: The full image or a part of the image.

Search in the database:

Search in a 5x5 degrees sky window.

Search in a 30x30 degrees sky window.

Search in a spherical ring.

Search in the full sky.

Search in a 2x2 degrees sky window for any image angle.

Search in a 40x40 minutes sky window for any image angle.

Proprietary database:

The idea was to built a database containing:

- the 5 brightest stars in every 15x10 arc minutes sky window.

- the pre-calculated (smaller than 18 arc minutes) distances between these stars.

Stars in the database:            8 155 625

Distances in the database:    60 332 797

Size of the database:  about  800 MBytes.

There are some 'holes' without stars in the database.

Equatorial mounts are supported:     Yes

Altazimuth mounts are no yet supported.

Calibration:  Using the pixel size and the image inclination.

Communication with others programs:

Synchronizes :

with  Cartes du ciel - Sky Chart by Patrick Chevalley,

with  COR by Cristóbal García,

with  PicGoto by  Angel Caparrós.

The program has a DDE server.

Communication with telescopes using the ASCOM plateform.

ERROR MESSAGES:

"Too many distances" :  The program searches in the data base the distances that are "similar" to the distances between the image stars. Whenever more than 15000 distances are found, the error message "Too many distances" is displayed.

Cause:  possibly the search parameters are too large.

Solution:  try with the normal parameters:

Distances percent error: 2

Inclination error : 1

Searching Window:  width :  5    height : 5

GLOSSARY:

calibrate: to adjust the program parameters for a particular pixel size and image orientation.

image angle:  the angle between the Oy image axis and the South-North direction in the image. For example if the North is pointing up and right by 30 degrees (like one hour in the clock ), the image angle should be +30 degrees.

compass:  when a solution is found, the four cardinal points are drawn on the image.

contour: a closed line formed by pixels with brightness greather or equal to the threshold.

distance: the angle between two stars.

Elbrus : means "Sidereal compass locating element" ( Elemento Localizador por BRUjula Sidérea in spanish) .

image centre: the coordinates of the centre of the image as calculated by the program.

manual mode: when the analysis of the image is launched manually by clicking the 'Analyse' button.

perimeter: the lenght in pixels of  the star contour. Too long perimeters are ignored.

polygon: here, the geometrical figure formed by stars. One star in each vertex.

rectangle ( the analysis rectangle): a rectangle drawn by moving the mouse over the image with the left button down. This rectangle defines the image zone from where the stars should be picked out for the analysis. When no rectangle is drawn, the full image is considered.

searching centre: the aproximate point we think we are pointing to. The search is done around this point.

searching window: the sky region around the "searching centre" where we seek the distances.

side: a line between two stars in a polygon. Here, the same as distance. Each side distance is searched in the distances database. The more sides in a polygon, the best the solution. For a 10 stars solution, the maximal number of sides is 45.

solution: When we find a polygon with enough sides, we say that we have found a solution.

target: the point we want to go to. When a solution is found, Elbrus draws a line with origin in the centre of the image and the end at the target point. This is a simple visual indicator of the telescope position.

threshold: at the beginning of the scanning process, pixels with intensity greater than the threshold are supposed to belong to a star.

wait-for mode: images are automatically analyzed as they come to the image folder.

End of the Help text.